GUN BOX NOTES

- Plane wood down to ½ inch thick.

- Layout and cut all pieces to plan. Fit up pieces to insure proper mating.

- Prior to assembly, layout and drill pilot holes on INSIDE of left side panels for slides per drawing. If using only one (1) drawer, eliminate the bottom drawer.

- Assemble case complete maintaining grain flow from left side to top right side.

- If using suggested handle, layout and drill pilot holes for wood screws.

- Measure 1½ inches from front of case. Using a thin blade band saw, cut through the case, completely. Sand cut edges until smooth.

- For additional strength, it is suggested that case clamps be installed on inside top sections connecting top panel to side panel and top panel to rear panel.

- Clamp the 2 sections together and template the hinge per plan. Mortise area 1/16” deep by 1-1/16” wide by 14” long.  Drill pilot holes for wood screws. Remove clamps.

- Route all outer edges of case with 5/16 inch radius.

- Measure inside of case for drawers.  Adjust width of drawers to ensure that ½ inch exits on both sides of drawers for slides and rails.  Bull nose (router ¼”) the top pieces of drawers for aesthetic looks.  Fabricate drawers complete per plan.  Note: If slides are not going to be used, add 7/8” to width of drawers. Notch left and right sides of rear section of drawers to accommodate inside panel rails.

- Apply several coats of spray lacquer to all parts of the case and drawers.  Follow directions per manufacturer’s instructions. Let dry for minimum of 48 hours.

- Layout and drill pilot holes for latches and case corners.  Install all hardware.

TOLERANCES

Case      +1/32 -0

Drawers +o -1/32